# SIMATS ENGINEERING

## ASSESSMENT TOOL 1

COURSE NAME:	CSA07 COURSE CODE: Computer Networks

COURSE OUTCOME COVERED

CO1: Apply the functions of OSI layer in enabling reliable data transmission across networks. (BL2, BL3)

Assessment Tool Weightage: 40%

Annexure A

SHORT ANSWER TEST

Code of Conduct:

I Bhoomika s(192524291)	(Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

Annexure A

Short Answer Test

- A company needs to transfer a 150 MB design file from a client computer to a remote server over a network with a bandwidth of 50 Mbps. The Transport Layer divides the file into 1500-byte segments, while the Data Link Layer encapsulates each segment into frames by adding 40 bytes of header and trailer. Calculate the total number of segments and frames transmitted, the total transmission overhead introduced by the Data Link Layer, and estimate the total transmission time (ignoring propagation delay). Explain how the Transport and Data Link layers ensure reliable communication.

Ans: Given:

- File size = 150 MB = 150 × 1,000,000 = 150,000,000 bytes

- Transport Layer segment size = 1500 bytes

- Data Link Layer overhead per frame = 40 bytes

- Bandwidth = 50 Mbps = 50,000,000 bits/second

- Assumption: One segment is encapsulated into one frame.

1. Number of Segments

Number of Segments=File SizeSegment Size
=150,000,0001500=100,000 segments

Answer: 100,000 segments

2. Number of Frames

Each segment is placed into one frame.

Number of Frames=100,000

Answer: 100,000 frames

3. Total Data Link Layer Overhead

Overhead per frame = 40 bytes

Total Overhead=100,000×40
=4,000,000 bytes

= 4 MB

Answer: 4,000,000 bytes (4 MB)

4. Total Data Transmitted

Original file:

150,000,000 bytes

Add overhead:

150,000,000+4,000,000=154,000,000 bytes

Convert to bits:

154,000,000×8=1,232,000,000 bits

5. Transmission Time

Transmission Time=Total BitsBandwidth
=1,232,000,00050,000,000
=24.64 seconds

Estimated Transmission Time = 24.64 seconds

- A Transport Layer protocol uses a sliding window size of 6 frames. Each frame carries 1024 bytes of user data. If the sender transmits 48 frames, calculate the number of transmission windows required. If one frame in every window is lost and must be retransmitted, determine the total number of retransmissions. Explain how flow control improves communication efficiency.

Ans: Given:

- Sliding window size = 6 frames

- Data per frame = 1024 bytes

- Total frames to transmit = 48 frames

1. Number of transmission windows required

Each window can send 6 frames.

Number of windows=Total framesWindow size=486=8

Answer: 8 transmission windows

2. Total number of retransmissions

It is given that one frame in every window is lost and must be retransmitted.

- Number of windows = 8

- Lost frames per window = 1

Total retransmissions=8×1=8

Answer: 8 frame retransmissions

3. How flow control improves communication efficiency

Flow control is a mechanism that regulates the rate of data transmission between the sender and receiver so that the receiver is not overwhelmed.

Benefits of flow control:

- Prevents the receiver's buffer from overflowing.

- Ensures smooth and reliable data transfer.

- Reduces packet/frame loss caused by excessive transmission speed.

- Minimizes unnecessary retransmissions.

- Improves overall network throughput and communication efficiency.

- A 12,000-byte IP packet is transmitted through a router where the Maximum Transmission Unit (MTU) is 1500 bytes. Assuming each fragment requires a 20-byte IP header, calculate the total number of fragments created and the total header overhead introduced. Explain how the Network Layer ensures successful delivery of fragmented packets to the destination.

Ans: Given:

- IP packet size = 12,000 bytes

- MTU = 1,500 bytes

- IP header per fragment = 20 bytes

Step 1: Calculate maximum data per fragment

Each fragment must include a 20-byte IP header, so the maximum data carried by each fragment is:

1500-20=1480 bytes

Step 2: Calculate the number of fragments

Number of fragments=120001480=8.11=9

Answer: 9 fragments

Step 3: Calculate the total header overhead

Each of the 9 fragments has a 20-byte IP header.

Total header overhead=9×20=180 bytes

Answer: 180 bytes

- A Transport Layer protocol uses a sliding window size of 6 frames. Each frame carries 1024 bytes of user data. If the sender transmits 48 frames, calculate the number of transmission windows required. If one frame in every window is lost and must be retransmitted, determine the total number of retransmissions. Explain how flow control improves communication efficiency.

Ans: Given:

- Sliding window size = 6 frames

- Data per frame = 1024 bytes

- Total frames to transmit = 48 frames

Step 1: Calculate the number of transmission windows

Each transmission window can send 6 frames.

Number of windows=Total framesWindow size=486=8

Answer: 8 transmission windows

Step 2: Calculate the total number of retransmissions

One frame is lost in each window and must be retransmitted.

- Number of windows = 8

- Lost frames per window = 1

Total retransmissions=8×1=8

Answer: 8 frame retransmissions

Step 3: Explain how flow control improves communication efficiency

Flow control is a Transport Layer mechanism that controls the rate at which the sender transmits data so that the receiver can process it without being overloaded.

Benefits of flow control:

- Prevents receiver buffer overflow.

- Matches the sender's transmission speed with the receiver's processing speed.

- Reduces frame loss and unnecessary retransmissions.

- Improves network throughput and communication reliability.

- Ensures efficient and smooth data transfer.

- A multimedia application transfers a 600 MB video file. The Session Layer inserts a synchronization checkpoint after every 60 MB of transmitted data. During transmission, a failure occurs after 390 MB has been transferred. Determine the number of checkpoints created before failure and calculate the amount of data that must be retransmitted after recovery. Explain the importance of synchronization in the Session Layer.

Ans: Given:

- Video file size = 600 MB

- Synchronization checkpoint after every 60 MB

- Failure occurs after 390 MB

Step 1: Calculate the number of checkpoints created before failure

A checkpoint is inserted every 60 MB.

Number of checkpoints=39060=6

The checkpoints are created at:

- 60 MB

- 120 MB

- 180 MB

- 240 MB

- 300 MB

- 360 MB

Answer: 6 checkpoints

Step 2: Calculate the data to be retransmitted

The last checkpoint was at 360 MB.

Failure occurred at 390 MB.

Data to retransmit=390-360=30 MB

Answer: 30 MB

Step 3: Importance of synchronization in the Session Layer

Synchronization in the Session Layer helps maintain reliable communication by inserting checkpoints during data transfer.

Importance:

- Allows data transmission to resume from the last checkpoint after a failure.

- Prevents retransmission of the entire file.

- Saves time and network bandwidth.

- Improves communication reliability and efficiency.

- Supports recovery from interruptions or network failures.

- Before transmission, the Presentation Layer compresses a 900 MB file by 40%. The compressed data is encrypted, adding 5% overhead to the compressed size. Calculate the compressed file size, the encrypted file size, and the total percentage reduction compared to the original file. Explain how the Presentation Layer improves transmission efficiency and security.

Ans: Given:

- Original file size = 900 MB

- Compression = 40%

- Encryption overhead = 5% of the compressed size

Step 1: Calculate the compressed file size

A 40% compression means the file size is reduced by 40%.

Compressed size=900-(40%×900)
=900-360=540 MB

Answer: 540 MB

Step 2: Calculate the encrypted file size

Encryption adds 5% overhead to the compressed size.

Overhead=5%×540=27 MB
Encrypted file size=540+27=567 MB

Answer: 567 MB

Step 3: Calculate the total percentage reduction

Reduction=900-567=333 MB
Percentage reduction=333900×100=37%

Answer: 37% reduction

- An FTP application transfers 3 GB of data over a network with an effective throughput of 120 Mbps. The Application Layer divides the data into requests of 3 MB each. Calculate the total number of requests generated and estimate the total transmission time in seconds. Discuss how the Application Layer provides reliable services to users.

Ans: Given:

- Total data = 3 GB

- Throughput = 120 Mbps

- Request size = 3 MB

Assume 1 GB = 1000 MB (common networking convention).

Step 1: Calculate the total number of requests

3 GB=3000 MB
Number of requests=3000 MB3 MB=1000

Answer: 1000 requests

Step 2: Calculate the transmission time

Convert 3 GB to megabits (Mb):

3000 MB×8=24000 Mb

Now,

Transmission Time=Total Data (Mb)Throughput (Mbps)=24000120=200 seconds

Answer: 200 seconds

- A packet experiences the following processing delays at different OSI layers: Application Layer = 4 ms, Transport Layer = 3 ms, Network Layer = 5 ms, Data Link Layer = 2 ms, and Physical Layer = 1 ms. The propagation delay is 18 ms, and the transmission delay is 12 ms. Calculate the total end-to-end delay experienced by the packet and explain how each layer contributes to the communication process.

Ans: Given:

- Application Layer delay = 4 ms

- Transport Layer delay = 3 ms

- Network Layer delay = 5 ms

- Data Link Layer delay = 2 ms

- Physical Layer delay = 1 ms

- Propagation delay = 18 ms

- Transmission delay = 12 ms

Step 1: Calculate the total processing delay

Processing Delay=4+3+5+2+1=15 ms

Step 2: Calculate the total end-to-end delay

Total Delay=Processing Delay+Propagation Delay+Transmission Delay
=15+18+12=45 ms

Answer: 45 ms

- A network transfers 80 MB of useful data using frames of 1600 bytes, where each frame contains 80 bytes of protocol overhead added by different OSI layers.

Calculate the total number of frames required, the total overhead transmitted, and the transmission efficiency as a percentage. Explain how protocol overhead affects network performance.

Ans: Given:

- Useful data = 80 MB

- Frame size = 1600 bytes

- Protocol overhead per frame = 80 bytes

Assume 1 MB = 1,000,000 bytes (common networking convention).

Step 1: Calculate useful data per frame

Useful data per frame=1600-80=1520 bytes

Step 2: Calculate the total number of frames

80 MB=80,000,000 bytes
Number of frames=80,000,0001520=52631.58=52,632

Answer: 52,632 frames

Step 3: Calculate the total overhead transmitted

Total overhead=52,632×80=4,210,560 bytes

Answer: 4,210,560 bytes (≈ 4.21 MB)

Step 4: Calculate the transmission efficiency

Transmission Efficiency=Useful DataUseful Data + Overhead×100
=80,000,00080,000,000+4,210,560×100
≈95%

Answer: Approximately 95%

- A hospital transfers a patient's medical record of 240 MB through an IoT-enabled healthcare network. The Presentation Layer compresses the data by 30%, the Transport Layer divides the compressed data into 1400-byte segments, and the Data Link Layer adds 60 bytes of overhead to each frame. Calculate the compressed file size, the total number of segments transmitted, the total protocol overhead introduced, and the final amount of data transmitted across the Physical Layer. Explain how multiple OSI layers work together to ensure secure and reliable communication.

Ans: Given:

- Original medical record size = 240 MB

- Compression = 30%

- Transport Layer segment size = 1400 bytes

- Data Link Layer overhead = 60 bytes per frame

Assume 1 MB = 1,000,000 bytes (networking convention).

Step 1: Calculate the compressed file size

Compression reduces the file size by 30%.

Compressed size=240-(30%×240)
=240-72=168 MB

Answer: 168 MB

Step 2: Calculate the total number of segments

Convert the compressed file size to bytes:

168 MB=168,000,000 bytes
Number of segments=168,000,0001400=120000=120,000

Answer: 120,000 segments

Step 3: Calculate the total protocol overhead

Each segment has 60 bytes of Data Link Layer overhead.

Total overhead=120,000×60=7,200,000 bytes
=7.2 MB

Answer: 7,200,000 bytes (7.2 MB)

Step 4: Calculate the final amount of data transmitted

Final data transmitted=168 MB+7.2 MB=175.2 MB

Answer: 175.2 MB

RUBRICS FOR CALCULATION

| Criteria | Weightag e | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understandin g of Concepts | 25% | Demonstrates a thorough, accurate understandin g of concepts | Good understanding with minor gaps | Basic understanding with some misconception s | Poor understanding; major misconception s |
| Explanation | 25% | Detailed, well-explained answers with relevant examples | Clear explanation with adequate details | Limited explanation; lacks depth | Poor or unclear explanation |
| Application | 20% | Effectively applies concepts with strong analytical ability | Applies concepts with minor errors | Limited application with weak analysis | No application or incorrect analysis |
| Organization | 15% | Well-structured answers with logical flow | Organized with minor issues in flow | Basic structure, lacks clarity | Poor organization, incoherent answers |

|  |  | and coherence |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Accuracy | 10% | Completely accurate and covers all required points | Mostly accurate with minor omissions | Partially correct with missing points | Incorrect answers with major gaps |
| Clarity | 5% | Clear, neat, professional presentation | Understandabl e with minor issues | Limited clarity, readability issues | Poorly written, difficult to understand |